Department of Electronics and Electrical

Communications Engineering

Faculty of Engineering

Cairo University

[DFT & OFDM system project]

4th Year

1st Semester – Academic year 2024/2025

Presented by

| Name | SEC | BN | ID |
| --- | --- | --- | --- |
| أحمد عادل يونس سيد | 1 | 17 | 9213073 |
| كريم أيمن محمد فخر الدين | 3 | 22 | 9210836 |

Under supervision of

Prof. Mohamed Khairy

Eng / Mohamed Nady

- Problem 1: Execution time of DFT and FFT

- In this section, it is required to compare between the execution time of Discrete Fourier Transform (DFT) and Fast Fourier Transform (FFT) where the discrete Fourier transform X(k) represented by this equation:

clc clear close all; %% First part (Execution time of DFT and FFT) L = 8192; x = rand(1, L); % Xi[n] random signal % Measure execution time for DFT {x[n]} tic; X_dft = DFT(x); dft_time = toc; fprintf('Execution Time for DFT = %.4f seconds \n', dft_time); % Measure execution time for FFT {x[n]} tic; X_fft = fft(x); % Calculate fast fourier transform directly using built-in function fft() fft_time = toc; fprintf('Execution Time for FFT = %.4f seconds\n', fft_time); %% DFT implementation Function function X = DFT(x) N = length(x); X = zeros(1, N); for k = 0:N-1 for n = 0:N-1 X(k+1) = X(k+1) + x(n+1) * exp(-1j * 2 * pi * n * k / N); end end end

- After simulating the code, we found that the execution time of DFT function is much higher than FFT built-in function as shown in figure (1). Therefore, FFT offers superior performance with respect to the execution time.

Figure (1): Simulation result

- Problem 2: Bit-error rate performance for BPSK & QPSK & 16-QAM over Rayleigh flat fading channel

clc clear close all; %% --------- Generating the Random Data Sequence-----------%% Number_of_Bits = 120000; Eb = 1; % Eb represent the bit eneregy Eb_QAM = 2.5; SNR_range_db = (-5:15); % where SNR_range = Eb/No SNR_range_linear = 10 .^ (SNR_range_db/10); No = Eb ./ SNR_range_linear; Random_Data = randi([0 1] , 1 , Number_of_Bits); % generate all the random bits %% ----------BPSK Modulation Scheme --------------%% % ----------------Declare BPSK (Nocode)--------------% BPSK = Mapper(Random_Data,1,Eb); % Symbol_Bits = 1 , Eb = 1 % -----------BPSK Channel stage (No code)-----------% BPSK_recieved = zeros(1,length(BPSK)); BER_BPSK = zeros(1,length(SNR_range_db)); h_BPSK = (1/sqrt(2)) .* complex(randn(1,length(BPSK)) , randn(1,length(BPSK))); % channel with zero mean & variance = 1/2 for i = 1 : length(SNR_range_db) noise_BPSK = sqrt(No(i)/2).* complex(randn(1,length(BPSK)) , randn(1,length(BPSK))) ; BPSK_recieved = (BPSK .* h_BPSK + noise_BPSK)./ h_BPSK; BPSK_demapped = DeMapper(BPSK_recieved,1); % calling the demapper function %---------BER calculation for the BPSK Scheme(No code)---------% error_bits_BPSK = 0; for j = 1 : length(Random_Data) if(BPSK_demapped(j) ~= Random_Data(j)) error_bits_BPSK = error_bits_BPSK + 1; end end BER_BPSK(1,i) = error_bits_BPSK/Number_of_Bits; end % ----------------Declare BPSK (Rep Code)--------------% BPSK_rep_bstream = SymbolRepetitionEncoder(Random_Data,1); BPSK_rep = Mapper(BPSK_rep_bstream,1,Eb); % -----------BPSK Channel stage (Rep code)-----------% BPSK_rep_recieved = zeros(1,length(BPSK_rep)); BER_BPSK_rep = zeros(1,length(SNR_range_db)); h_BPSK_rep = (1/sqrt(2)) .* complex(randn(1,length(BPSK_rep)) , randn(1,length(BPSK_rep))); for i = 1 : length(SNR_range_db) noise_BPSK_rep = sqrt(No(i)/2).* complex(randn(1,length(BPSK_rep)) , randn(1,length(BPSK_rep))) ; BPSK_rep_recieved = (BPSK_rep .* h_BPSK_rep + noise_BPSK_rep)./ h_BPSK_rep; BPSK_rep_demapped = DeMapper(BPSK_rep_recieved,1); % calling the demapper function BPSK_rep_decoded = majority_decoder(BPSK_rep_demapped,1); % Apply the majority decoder function %---------BER calculation for the BPSK Scheme(Rep code)---------% error_bits_BPSK = 0; for j = 1 : length(Random_Data) if(BPSK_rep_decoded(j) ~= Random_Data(j)) error_bits_BPSK = error_bits_BPSK + 1; end end BER_BPSK_rep(1,i) = error_bits_BPSK/Number_of_Bits; end % ------------------plotting the BER for BPSK--------------% figure; semilogy(SNR_range_db , BER_BPSK,'k','linewidth',1.5); hold on; semilogy(SNR_range_db , BER_BPSK_rep,'r','linewidth',1.5); %hold off; title('BER of BPSK Modulation Scheme'); xlabel('Eb/No (dB)'); grid on; legend('BER without repetition coding','BER with repetition coding','Location','southwest') %% -----------QPSK Modulation Scheme ------------%% % -----------------Declare QPSK (No code)-------------% QPSK = Mapper(Random_Data,2,Eb); % ------------------QPSK channel stage(No code)------------% QPSK_recieved = zeros(1,length(QPSK)); BER_QPSK = zeros(1,length(SNR_range_db)); h_QPSK = (1/sqrt(2)) .* complex(randn(1,length(QPSK)) , randn(1,length(QPSK))); for i = 1 : length(SNR_range_db) noise_QPSK = sqrt(No(i)/2).* complex(randn(1,length(QPSK)) , randn(1,length(QPSK))); QPSK_recieved = (QPSK .* h_QPSK + noise_QPSK) ./ h_QPSK; QPSK_demapped = DeMapper(QPSK_recieved,2); % Calling demapper function %----------BER calculation for QPSK (No code)--------------% error_bits_QPSK = 0; inc_var = 1; for j = 1 : length(Random_Data)/2 for k = 1 : 2 if(QPSK_demapped(j,k) ~= Random_Data(inc_var)) error_bits_QPSK = error_bits_QPSK + 1; end inc_var = inc_var + 1; end end BER_QPSK(1,i) = error_bits_QPSK/Number_of_Bits; end % -----------------Declare QPSK (Rep code)-------------% QPSK_rep_bstream = SymbolRepetitionEncoder(Random_Data,2); QPSK_rep = Mapper(QPSK_rep_bstream,2,Eb); % ------------------QPSK channel stage(Rep code)-----------------% QPSK_rep_recieved = zeros(1,length(QPSK_rep)); BER_QPSK_rep = zeros(1,length(SNR_range_db)); h_QPSK_rep = (1/sqrt(2)) .* complex(randn(1,length(QPSK_rep)) , randn(1,length(QPSK_rep))); for i = 1 : length(SNR_range_db) noise_QPSK_rep = sqrt(No(i)/2).* complex(randn(1,length(QPSK_rep)) , randn(1,length(QPSK_rep))); QPSK_rep_recieved = (QPSK_rep .* h_QPSK_rep + noise_QPSK_rep) ./ h_QPSK_rep; QPSK_demapped_rep = DeMapper(QPSK_rep_recieved,2); % Calling demapper function QPSK_decoded_rep = majority_decoder(reshape(QPSK_demapped_rep',1,[]),2); numSymbols = length(QPSK_decoded_rep) / 2; QPSK_decoded_rep = inverseReshape(QPSK_decoded_rep,[numSymbols,2]); %-------------BER calculation for QPSK (Rep code)-------------% error_bits_QPSK = 0; inc_var = 1; for j = 1 : length(Random_Data)/2 for k = 1 : 2 if(QPSK_decoded_rep(j,k) ~= Random_Data(inc_var)) error_bits_QPSK = error_bits_QPSK + 1; end inc_var = inc_var + 1; end end BER_QPSK_rep(1,i) = error_bits_QPSK/Number_of_Bits; end % -----------------plotting the BER for QPSK------------------% figure; semilogy(SNR_range_db , BER_QPSK ,'k','linewidth',1.5); hold on; semilogy(SNR_range_db , BER_QPSK_rep ,'r','linewidth',1.5); title('BER of QPSK Modulation Scheme'); xlabel('Eb/No (dB)'); grid on; legend('BER without repetition coding','BER with repetition coding','Location','southwest') %% -----------16-QAM Modulation Scheme ---------------%% % -----------Declare 16-QAM(No code)------------------% MQAM = Mapper(Random_Data,4,Eb_QAM); %---------------16-QAM channel stage(No code)------------% MQAM_recieved = zeros(1,length(MQAM)); BER_MQAM = zeros(1,length(SNR_range_db)); h_MQAM = (1/sqrt(2)) .* complex(randn(1,length(MQAM)) , randn(1,length(MQAM))); for i = 1 : length(SNR_range_db) noise_MQAM = sqrt(No(i)/2).* complex(randn(1,length(MQAM)) , randn(1,length(MQAM))); MQAM_recieved = (MQAM .* h_MQAM + noise_MQAM) ./ h_MQAM; MQAM_demapped = DeMapper(MQAM_recieved,4); % Calling demapper function %----------BER calculation for 16-QAM scheme(No code)----------% error_bits_MQAM = 0; inc_var = 1; for j = 1 : length(Random_Data)/4 for k = 1 : 4 if(MQAM_demapped(j,k) ~= Random_Data(inc_var)) error_bits_MQAM = error_bits_MQAM + 1; end inc_var = inc_var + 1; end end BER_MQAM(1,i) = error_bits_MQAM/Number_of_Bits; end % -----------Declare 16-QAM(Rep Code)-----------------% MQAM_rep_bstream = SymbolRepetitionEncoder(Random_Data,4); MQAM_rep = Mapper(MQAM_rep_bstream,4,Eb_QAM); %---------------16-QAM channel stage(Rep code)------------% MQAM_rep_recieved = zeros(1,length(MQAM_rep)); BER_MQAM_rep = zeros(1,length(SNR_range_db)); h_MQAM_rep = (1/sqrt(2)) .* complex(randn(1,length(MQAM_rep)) , randn(1,length(MQAM_rep))); for i = 1 : length(SNR_range_db) noise_MQAM_rep = sqrt(No(i)/2).* complex(randn(1,length(MQAM_rep)) , randn(1,length(MQAM_rep))); MQAM_rep_recieved = (MQAM_rep .* h_MQAM_rep + noise_MQAM_rep) ./ h_MQAM_rep; MQAM_demapped_rep = DeMapper(MQAM_rep_recieved,4); % Calling demapper function MQAM_decoded_rep = majority_decoder(reshape(MQAM_demapped_rep',1,[]),4); numSymbols = length(MQAM_decoded_rep) / 4; MQAM_decoded_rep = inverseReshape(MQAM_decoded_rep,[numSymbols,4]); %------------BER calculation for 16-QAM scheme(Rep code)---------% error_bits_MQAM = 0; inc_var = 1; for j = 1 : length(Random_Data)/4 for k = 1 : 4 if(MQAM_decoded_rep(j,k) ~= Random_Data(inc_var)) error_bits_MQAM = error_bits_MQAM + 1; end inc_var = inc_var + 1; end end BER_MQAM_rep(1,i) = error_bits_MQAM/Number_of_Bits; end % -----------------plotting the BER for 16-QAM---------------% figure; semilogy(SNR_range_db , BER_MQAM,'k','linewidth',1.5); hold on; semilogy(SNR_range_db , BER_MQAM_rep,'r','linewidth',1.5); title('BER of 16-QAM Modulation Scheme'); xlabel('Eb/No (dB)'); grid on; legend('BER without repetition coding','BER with repetition coding','Location','southwest') %% ----------Mapper function-----------%% function mapped_constellations = Mapper(inputstream, symbolBits,Eb) % Eb represents the info energy Number_of_Bits = length(inputstream); mapped_constellations = zeros(1,Number_of_Bits/symbolBits); switch symbolBits case 1 % BPSK mapping mapped_constellations = sqrt(Eb) * (2 * inputstream - 1); case 2 % QPSK mapping for i = 1:symbolBits:Number_of_Bits if(inputstream(i) == 0 && inputstream(i+1) == 0 ) mapped_constellations((i+1)/2) = sqrt(Eb) * complex(-1,-1); elseif(inputstream(i) == 0 && inputstream(i+1) == 1) mapped_constellations((i+1)/2) = sqrt(Eb) * complex(-1,1); elseif(inputstream(i) == 1 && inputstream(i+1) == 0) mapped_constellations((i+1)/2) = sqrt(Eb) * complex(1,-1); elseif(inputstream(i) == 1 && inputstream(i+1) == 1) mapped_constellations((i+1)/2) = sqrt(Eb) * complex(1,1); end end case 4 % 16-QAM mapping real_MQAM = zeros(1,length(inputstream)/symbolBits); img_MQAM = zeros(1,length(inputstream)/symbolBits); for i = 1:symbolBits:Number_of_Bits if(inputstream(i) == 0 && inputstream(i+1) == 0) % First two bits control the real part real_MQAM((i+3)/4) = -3; elseif(inputstream(i) == 0 && inputstream(i+1) == 1) real_MQAM((i+3)/4) = -1; elseif(inputstream(i) == 1 && inputstream(i+1) == 1) real_MQAM((i+3)/4) = 1; elseif(inputstream(i) == 1 && inputstream(i+1) == 0) real_MQAM((i+3)/4) = 3; end if(inputstream(i+2) == 0 && inputstream(i+3) == 0) % Second two bits control the imaginary img_MQAM((i+3)/4) = -3; elseif(inputstream(i+2) == 0 && inputstream(i+3) == 1) img_MQAM((i+3)/4) = -1; elseif(inputstream(i+2) == 1 && inputstream(i+3) == 1) img_MQAM((i+3)/4) = 1; elseif(inputstream(i+2) == 1 && inputstream(i+3) == 0) img_MQAM((i+3)/4) = 3; end mapped_constellations((i+3)/4) = sqrt(Eb) * complex(real_MQAM((i+3)/4),img_MQAM((i+3)/4)); end end end %% -----------Repeatition encoder------------- %% function encodedStream = SymbolRepetitionEncoder(inputStream, symbolBits) % Reshape the input stream into symbols numSymbols = length(inputStream) / symbolBits; symbols = reshape(inputStream, symbolBits, numSymbols)'; repeatedSymbols = repelemsymbols, 3, 1; % Repeats each symbol by 3 times % Flatten the repeated symbols back into a binary stream encodedStream = repeatedSymbols'; encodedStream = encodedStream(:)'; end %% --------- DeMapper Function-------------%% function demapped_data = DeMapper (received_data ,symbolBits) Eb = 1; Eb_QAM = 1; demapped_data = zeros(1,length(received_data)); if symbolBits == 1 % BPSK case BPSK_table = sqrt(Eb) * [complex(-1,0), complex(1,0)]; for j = 1 : length(received_data) [~ , Min_index] = min(abs(received_data(j) - BPSK_table)); demapped_data(j) = Min_index - 1; end demapped_data = de2bi(demapped_data,1,'left-msb'); elseif symbolBits == 2 % QPSK case QPSK_table = sqrt(Eb) * [complex(-1,-1), complex(-1,1), complex(1,-1), complex(1,1)]; for j = 1 : length(received_data) [~ , Min_index] = min(abs(received_data(j) - QPSK_table)); demapped_data(j) = Min_index - 1; end demapped_data = de2bi(demapped_data,2,'left-msb'); elseif symbolBits == 4 % 16-QAM case MQAM_table = sqrt(Eb_QAM)*[complex(-3,-3), complex(-3,-1), complex(-3,3), complex(-3,1), complex(-1,-3), complex(-1,-1), complex-1,3, complex-1,1, complex3,-3, complex3,-1, complex3,3, complex3,1,complex1,-3, complex(1,-1), complex(1,3), complex(1,1)]; for j = 1 : length(received_data) [~ , Min_index] = min(abs(received_data(j) - MQAM_table)); demapped_data(j) = Min_index - 1; end demapped_data = de2bi(demapped_data,4,'left-msb'); end end %% ------------- Decoding Function ------------ %% function decoded_bitstream = majority_decoder (encoded_bitstream , symbolBits ) hash_table = zeros(1,2^symbolBits); iterations = 0; j=1; decoded_bitstream = zeros(1,length(encoded_bitstream)/3); % Assume 3-repeatition code for i = 1 : symbolBits : length(encoded_bitstream) iterations = iterations + 1 ; % Convert the symbol into the correspoding decimal into a hashtable % to see the no. of occurrences of such symbol for decision index = bin2dec(num2str((encoded_bitstream(i:i+symbolBits-1))))+1; hash_table(index) = hash_table(index) + 1; if(iterations==3) %decide based on the most no. of occurrences % then get the corrsponding binary symbol index_of_most_repeated_symbol=find(hash_table==max(hash_table),1); binaryStr = dec2bin(index_of_most_repeated_symbol-1, symbolBits); decoded_bitstream(j:j+symbolBits-1) = double(binaryStr)-'0'; j = j + symbolBits; hash_table = zeros(1,2^symbolBits); % Reset the hash table for the next set of symbols iterations = 0; end end end %% --------- Inverse reshape Function ---------- %% function originalMatrix = inverseReshape(rowVector, originalSize) % Reshape the row vector into the transposed matrix shape transposedMatrix = reshape(rowVector, fliplr(originalSize)); % Transpose the matrix to restore the original order originalMatrix = transposedMatrix.'; end

- The above code explains the steps of BER calculation over Rayleigh fading channel:

- Generate Random Bit stream bk = [0, 1, 0, 1, …].

- Generate BPSK symbols based on the bit stream  xk= [-Eb ,  Eb , -Eb ,  Eb , …]

and generate the QPSK & 16-QAM symbols based on their constellation.

Figure (2): Constellation diagram for BPSK, QPSK, 16-QAM

- Generate the complex channel vector (hr+jhi) and complex noise vector (nc+jns)

based on the distributions explained above.

- Compute the received symbol vector as follows:

yk= hr+jhik*xk+ nc+jnsk       where k= 0,1,2,….

- Compensate for the channel gain at the receiver (assuming the channel is known at the receiver), apply correlator and make hard decision decoding to estimate the transmitted bit stream (bk).

- Compute the bit-error rate (BER) for each SNR value [SNR= EbNo].

- Plot the BER against SNR.

- Repeat the above steps using a rate 1/3 repetition code. This is done by transmitting every “1” as three “1’s” and every “0” as three “0’s”.

- After simulating the code, we produce BER graph for each modulation scheme (BPSK, QPSK, 16-QAM) where each graph compares certain modulation scheme without any channel coding vs with channel coding (repetition 3 coding).

Figure (3): BER vs SNR for BPSK modulation scheme

Figure (4): BER vs SNR for QPSK modulation scheme

Figure (5): BER vs SNR for 16-QAM modulation scheme

- From the above graphs we notice that:

- The BER in case of repetition coding is better than BER with no coding and this is what we expect as in repetition coding, each bit is transmitted multiple times (e.g., three times for 3-repetition coding) so, the receiver uses majority decoding logic to decide the bit value.
single-bit error in any of the three bits still results in correct decoding using majority logic.

- BER for BPSK is very close to BER for QPSK and the reason behind that is the theoretical value of the BER for BPSK equal to that of QPSK

BERBPSK= BERQPSK =12*erfc(SNR)

- Problem 3: OFDM system simulation

Channel

Add Cyclic extension

128-IFFT block

Mapper block

Interleave block

Coding block

Decyclic extension

128-FFT block

Demapper block

Deinterleave block

Decoding block

Figure (6): OFDM system block diagram

Data bits

- Role of each block in brief:

- Coding block  The input to the system is a stream of binary data bits is coded first (for channel coding “repetition” only). This coding can be applied to the data bits to improve BER.

- Interleave block  The data bits are interleaved to spread the impact of burst errors caused by channel fading. In our case we use interleaver with size (16*16) for QPSK, and the size of the interleaver is (32*16) for 16-QAM.

- Mapper block  The interleaved bits are mapped to complex symbols (e.g., QPSK, 16-QAM) using a constellation mapping scheme mentioned above in figure (2).

- IFFT block  The mapped symbols are grouped into blocks of 128 symbols and transformed into the time domain using an Inverse Fast Fourier Transform function ifft ().

- Cyclic extension block  The last part of the IFFT output is appended to the beginning of the OFDM symbol, creating a cyclic prefix. Where the length of cyclic prefix = 25% of IFFT length (in our case cyclic prefix length = 32). This process helps mitigate Inter-Symbol Interference (ISI) caused by channel delay spread.

- Channel  The OFDM symbols are transmitted through the channel. Two channel models are considered, which are Rayleigh Flat Fading & Frequency Selective Fading.

- Receiver blocks  All blocks at the Rx perform the reverse operation performed by the transmitter blocks mentioned before.

clc clear close all; %% -----------QPSK & 16-QAM with No coding---------------%% Number_of_Bits = 435200; % 43520 , 435200 , 1044480 Random_Data = randi([0 1],1,Number_of_Bits); Eb = 1; Eb_QAM = 2.5 * Eb; SNR_range_db = (-5:15); %---------------------Interleaver------------------% for i=1:256:Number_of_Bits QPSK_intrlv(1,i:i+255) = matintrlv(Random_Data(1,i:i+255),16,16); % for QPSK Modulation end for i=1:512:Number_of_Bits QAM_intrlv(1,i:i+511) = matintrlv(Random_Data(1,i:i+511),32,16); % for 16-QAM Modulation end %------------------------Mapper----------------------% QPSK_mapped = Mapper(QPSK_intrlv,2,Eb); QAM_mapped = Mapper(QAM_intrlv,4,Eb_QAM); %-------------------------IFFT-----------------------% QPSK_IFFT = zeros(1,length(QPSK_mapped)); for i=1:128:length(QPSK_mapped) QPSK_IFFT(1,i:i+127) = ifft(QPSK_mapped(1,i:i+127),128); end QAM_IFFT = zeros(1,length(QAM_mapped)); for i=1:128:length(QAM_mapped) QAM_IFFT(1,i:i+127) = ifft(QAM_mapped(1,i:i+127),128); end %---------------------Cyclic Extension-----------------% count1 = 1; QPSK_CYC = []; % Initialize the QPSK_CYC matrix for i = 1:128:(Number_of_Bits/2) % Add cyclic prefix with size 32 (cyclic prefix length) and append to QPSK_IFFT to produce 160 bit block QPSK_cyclic(128+32) QPSK_CYC((160 * (count1 - 1) + 1):(count1 * 160)) = [QPSK_IFFT(1, ((i + 96):(i + 127))), QPSK_IFFT(1, (i:(i + 127)))]; count1 = count1 + 1; end count2 = 1; QAM_CYC = []; % Initialize the QAM_CYC matrix for i = 1:128:(Number_of_Bits/4) % Add cyclic prefix and append to QPSK_cyclic QAM_CYC((160 * (count2 - 1) + 1):(count2 * 160)) = [QAM_IFFT(1, ((i + 96):(i + 127))), QAM_IFFT(1, (i:(i + 127)))]; count2 = count2 + 1; end %-------------------------Channel---------------------% % outputs from Rayleigh flat fading channel QPSK_ch = Flat_channel(QPSK_CYC,SNR_range_db); QAM_ch = Flat_channel(QAM_CYC,SNR_range_db); % outputs from Frequency selective (FS) Fading channel QPSK_ch_FS = FS_channel(QPSK_CYC,SNR_range_db); QAM_ch_FS = FS_channel(QAM_CYC,SNR_range_db); %=======================At Reciever=========================% %--------------------De-Cyclic Extension--------------------% QPSK_DeCYC = Decyclic(QPSK_ch,SNR_range_db); % First case for QPSK [flat channel] QPSK_DeCYC_FS = Decyclic(QPSK_ch_FS,SNR_range_db); % Second case for QPSK [frequency selective channel] QAM_DeCYC = Decyclic(QAM_ch,SNR_range_db); % Third case for QAM [flat channel] QAM_DeCYC_FS = Decyclic(QAM_ch_FS,SNR_range_db); % Fourth case for QAM [frequency selective channel] %----------------FFT----------------% QPSK_FFT = FFT(QPSK_DeCYC,SNR_range_db); QPSK_FFT_FS = FFT(QPSK_DeCYC_FS,SNR_range_db); QAM_FFT = FFT(QAM_DeCYC,SNR_range_db); QAM_FFT_FS = FFT(QAM_DeCYC_FS,SNR_range_db); %----------------Demapper----------------% QPSK_demapped = DeMapper(QPSK_FFT,SNR_range_db,2,Eb); QPSK_demapped_FS = DeMapper(QPSK_FFT_FS,SNR_range_db,2,Eb); QAM_demapped = DeMapper(QAM_FFT,SNR_range_db,4,Eb_QAM); QAM_demapped_FS = DeMapper(QAM_FFT_FS,SNR_range_db,4,Eb_QAM); %-----------Deinterleaver------------------% QPSK_Deintrlv = Deinterleaver(QPSK_demapped,SNR_range_db,2); QPSK_FS_Deintrlv = Deinterleaver(QPSK_demapped_FS,SNR_range_db,2); QAM_Deintrlv = Deinterleaver(QAM_demapped,SNR_range_db,4); QAM_FS_Deintrlv = Deinterleaver(QAM_demapped_FS,SNR_range_db,4); %------------BER Calculation-----------------% BER_QPSK = compute_BER(QPSK_Deintrlv , Random_Data,SNR_range_db); BER_QPSK_FS = compute_BER(QPSK_FS_Deintrlv , Random_Data,SNR_range_db); BER_QAM = compute_BER(QAM_Deintrlv , Random_Data,SNR_range_db); BER_QAM_FS = compute_BER(QAM_FS_Deintrlv , Random_Data,SNR_range_db); %% ---------------QPSK & 16-QAM with Repeatition coding------------%% QPSK_rep = SymbolRepetitionEncoder(Random_Data,2); QAM_rep = SymbolRepetitionEncoder(Random_Data,4); %---------------------Interleaver (Repetition)------------------% for i=1:255:length(QPSK_rep) intrlv_out = matintrlv([QPSK_rep(1,i:i+254),0],16,16); % for QPSK Modulation with repeatiton if(i==1) QPSK_rep_intrlv = intrlv_out; else QPSK_rep_intrlv = [QPSK_rep_intrlv intrlv_out]; end end for i=1:510:length(QAM_rep) intrlv_out = matintrlv([QAM_rep(1,i:i+509),0,0],32,16); % for 16-QAM Modulation with repeatition if(i==1) QAM_rep_intrlv = intrlv_out; else QAM_rep_intrlv = [QAM_rep_intrlv intrlv_out]; end end %---------------Mapper (Repetition)----------------% QPSK_rep_mapped = Mapper(QPSK_rep_intrlv,2,Eb/sqrt(3)); % Same energy per info QAM_rep_mapped = Mapper(QAM_rep_intrlv,4,Eb_QAM/sqrt(3)); % Same energy per info %-------------IFFT (Repetition)----------------% QPSK_rep_IFFT = zeros(1,length(QPSK_rep_mapped)); for i=1:128:length(QPSK_rep_mapped) QPSK_rep_IFFT(1,i:i+127) = ifft(QPSK_rep_mapped(1,i:i+127),128); end QAM_IFFT = zeros(1,length(QAM_rep_mapped)); for i=1:128:length(QAM_rep_mapped) QAM_rep_IFFT(1,i:i+127) = ifft(QAM_rep_mapped(1,i:i+127),128); end %-------Cyclic Extension (Repetition)-------------% count3 = 1; QPSK_rep_CYC = []; % Initialize the QPSK_CYC matrix for i = 1:128:length(QPSK_rep_mapped) QPSK_rep_CYC((160 * (count3 - 1) + 1):(count3 * 160)) = [QPSK_rep_IFFT(1,((i + 96):(i + 127))), QPSK_rep_IFFT(1,(i:(i + 127)))]; count3 = count3 + 1; end count4 = 1; QAM_rep_CYC = []; for i = 1:128:length(QAM_rep_mapped) QAM_rep_CYC((160 * (count4 - 1) + 1):(count4 * 160)) = [QAM_rep_IFFT(1,((i + 96):(i + 127))), QAM_rep_IFFT(1,(i:(i + 127)))]; count4 = count4 + 1; end %----------------Channel---------------------% % outputs from Rayleigh flat fading channel QPSK_rep_ch = Flat_channel(QPSK_rep_CYC,SNR_range_db); % for QPSK QAM_rep_ch = Flat_channel(QAM_rep_CYC,SNR_range_db); % for QAM % outputs from Frequency selective (FS) Fading channel QPSK_rep_ch_FS = FS_channel(QPSK_rep_CYC,SNR_range_db); % for QPSK QAM_rep_ch_FS = FS_channel(QAM_rep_CYC,SNR_range_db); % for QAM %====================At Reciever=======================% %--------------De-Cyclic Extension----------------% % First case for QPSK [flat channel] QPSK_rep_DeCYC = Decyclic(QPSK_rep_ch,SNR_range_db); % Second case for QPSK [frequency selective channel] QPSK_rep_DeCYC_FS = Decyclic(QPSK_rep_ch_FS,SNR_range_db); % Third case for QAM [flat channel] QAM_rep_DeCYC = Decyclic(QAM_rep_ch,SNR_range_db); % Fourth case for QAM [frequency selective channel] QAM_rep_DeCYC_FS = Decyclic(QAM_rep_ch_FS,SNR_range_db); %-----------------FFT (Repitition)--------------------% QPSK_rep_FFT = FFT(QPSK_rep_DeCYC,SNR_range_db); % QPSK [Flat-Fading] QPSK_rep_FFT_FS = FFT(QPSK_rep_DeCYC_FS,SNR_range_db); % QPSK [Frequency Slective] QAM_rep_FFT = FFT(QAM_rep_DeCYC,SNR_range_db); % QAM [Flat-Fading] QAM_rep_FFT_FS = FFT(QAM_rep_DeCYC_FS,SNR_range_db); % QAM [Frequency Slective] %----------------Demapper (Repitition)----------------% QPSK_rep_demapped = DeMapper(QPSK_rep_FFT,SNR_range_db,2,Eb); % QPSK [Flat-Fading] QPSK_rep_demapped_FS = DeMapper(QPSK_rep_FFT_FS,SNR_range_db,2,Eb); % QPSK [Frequency Slective] QAM_rep_demapped = DeMapper(QAM_rep_FFT,SNR_range_db,4,Eb_QAM); % QAM [Flat-Fading] QAM_rep_demapped_FS = DeMapper(QAM_rep_FFT_FS,SNR_range_db,4,Eb_QAM); % QAM [Frequency Slective] %------------Deinterleaver(Repitition)----------------% QPSK_rep_Deintrlv = Deinterleaver(QPSK_rep_demapped,SNR_range_db,2); QPSK_rep_FS_Deintrlv = Deinterleaver(QPSK_rep_demapped_FS,SNR_range_db,2); QAM_rep_Deintrlv = Deinterleaver(QAM_rep_demapped,SNR_range_db,4); QAM_rep_FS_Deintrlv = Deinterleaver(QAM_rep_demapped_FS,SNR_range_db,4); % Remove the padding done at the interleaving stage at the Tx QPSK_rep_Deintrlv = pad_removal(QPSK_rep_Deintrlv,2); QPSK_rep_FS_Deintrlv = pad_removal(QPSK_rep_FS_Deintrlv,2); QAM_rep_Deintrlv = pad_removal(QAM_rep_Deintrlv,4); QAM_rep_FS_Deintrlv = pad_removal(QAM_rep_FS_Deintrlv,4); %-----------Majority Decoding------------------% for i=1:length(SNR_range_db) QPSK_rep_decoded(i,:) = majority_decoder(QPSK_rep_Deintrlv(i,:),2); QPSK_rep_FS_decoded(i,:) = majority_decoder(QPSK_rep_FS_Deintrlv(i,:),2); QAM_rep_decoded(i,:) = majority_decoder(QAM_rep_Deintrlv(i,:),4); QAM_rep_FS_decoded(i,:) = majority_decoder(QAM_rep_FS_Deintrlv(i,:),4); end %------------BER Calculation (Repitition)--------------% BER_QPSK_rep = compute_BER(QPSK_rep_decoded , Random_Data,SNR_range_db); BER_QPSK_rep_FS = compute_BER(QPSK_rep_FS_decoded, Random_Data,SNR_range_db); BER_QAM_rep = compute_BER(QAM_rep_decoded , Random_Data,SNR_range_db); BER_QAM_rep_FS = compute_BER(QAM_rep_FS_decoded , Random_Data,SNR_range_db); %------------Plotting BER vs SNR(Eb/No)---------------% figure; semilogy(SNR_range_db , BER_QPSK ,'k','linewidth',1.5); hold on; semilogy(SNR_range_db , BER_QPSK_rep ,'r','linewidth',1.5); xlabel('Eb/No (dB)'); grid on; title('BER of Flat fading channel QPSK'); legend('without coding','with Repitition coding','location','southwest') figure; semilogy(SNR_range_db , BER_QPSK_FS ,'k','linewidth',1.5); hold on; semilogy(SNR_range_db , BER_QPSK_rep_FS ,'r','linewidth',1.5); xlabel('Eb/No (dB)') grid on; title('BER of frequency selective channel QPSK'); legend('without coding','with Repitition coding','location','southwest') figure; semilogy(SNR_range_db , BER_QAM ,'k','linewidth',1.5); hold on; semilogy(SNR_range_db , BER_QAM_rep ,'r','linewidth',1.5); xlabel('Eb/No (dB)'); grid on; title('BER of Flat fading channel 16-QAM'); legend('without coding','with Repitition coding','location','southwest') figure; semilogy(SNR_range_db , BER_QAM_FS ,'k','linewidth',1.5); hold on; semilogy(SNR_range_db , BER_QAM_rep_FS ,'r','linewidth',1.5); xlabel('Eb/No (dB)'); grid on; title('BER of frequency selective channel 16-QAM'); legend('without coding','with Repitition coding','location','southwest') %% -----------------Repeatition encoder---------------- %% function encodedStream = SymbolRepetitionEncoder(inputStream, symbolBits) % Reshape the input stream into symbols numSymbols = length(inputStream) / symbolBits; symbols = reshape(inputStream, symbolBits, numSymbols)'; % Repeat each symbol 3 times repeatedSymbols = repelem(symbols, 3, 1); % Flatten the repeated symbols back into a binary stream encodedStream = repeatedSymbols'; encodedStream = encodedStream(:)'; end %% ----------Mapper function--------------%% function mapped_constellations = Mapper(inputstream, symbolBits, Eb) % Eb represents the info energy Number_of_Bits = length(inputstream); mapped_constellations = zeros(1,Number_of_Bits/symbolBits); switch symbolBits case 2 % QPSK mapping for i = 1:symbolBits:Number_of_Bits if(inputstream(i) == 0 && inputstream(i+1) == 0 ) mapped_constellations((i+1)/2) = sqrt(Eb) * complex(-1,-1); elseif(inputstream(i) == 0 && inputstream(i+1) == 1) mapped_constellations((i+1)/2) = sqrt(Eb) * complex(-1,1); elseif(inputstream(i) == 1 && inputstream(i+1) == 0) mapped_constellations((i+1)/2) = sqrt(Eb) * complex(1,-1); elseif(inputstream(i) == 1 && inputstream(i+1) == 1) mapped_constellations((i+1)/2) = sqrt(Eb) * complex(1,1); end end case 4 % 16-QAM mapping real_MQAM = zeros(1,length(inputstream)/symbolBits); img_MQAM = zeros(1,length(inputstream)/symbolBits); for i = 1:symbolBits:Number_of_Bits if(inputstream(i) == 0 && inputstream(i+1) == 0) % First two bits control the real part real_MQAM((i+3)/4) = -3; elseif(inputstream(i) == 0 && inputstream(i+1) == 1) real_MQAM((i+3)/4) = -1; elseif(inputstream(i) == 1 && inputstream(i+1) == 1) real_MQAM((i+3)/4) = 1; elseif(inputstream(i) == 1 && inputstream(i+1) == 0) real_MQAM((i+3)/4) = 3; end if(inputstream(i+2) == 0 && inputstream(i+3) == 0) % Second two bits control the imaginary img_MQAM((i+3)/4) = -3; elseif(inputstream(i+2) == 0 && inputstream(i+3) == 1) img_MQAM((i+3)/4) = -1; elseif(inputstream(i+2) == 1 && inputstream(i+3) == 1) img_MQAM((i+3)/4) = 1; elseif(inputstream(i+2) == 1 && inputstream(i+3) == 0) img_MQAM((i+3)/4) = 3; end mapped_constellations((i+3)/4) = sqrt(Eb) * complex(real_MQAM((i+3)/4),img_MQAM((i+3)/4)); end end end %% ---------Rayleigh flat fading channel Function--------------%% function Received_data = Flat_channel(cyclic_input,SNR_range_db) SNR_range_linear = zeros(1,length(SNR_range_db)); Received_data = zeros(length(SNR_range_db),length(cyclic_input)); for i = 1:length(SNR_range_db) SNR_range_linear(i) = 10 ^ (SNR_range_db(i)/10); for j = 1 : 160 : length(cyclic_input) Eavg = mean(abs(cyclic_input(j:j+159)).^2); h_channel = (1/sqrt(2)) .* complex(randn(1,160) , randn(1,160)); noise = sqrt(Eavg/(2*SNR_range_linear(i))) .* complex(randn(1,160) , randn(1,160)); Received_data(i,j:j+159) = (cyclic_input(j:j+159) .* h_channel + noise) ./ h_channel; end end end %% ------------Frequency selective fading channel Function----------%% function Received_data = FS_channel(cyclic_input,SNR_range_db) Cyclic_symbols = 160; Number_of_subchannels = 2; channel_length = Cyclic_symbols/Number_of_subchannels; gain = [0.5,0.5]; % FS gain of the subchannels (2 subchannels) (equal channel assignment) % Each Rayleigh subchannel is scaled with different gain ch1_time = gain(1)*(1/sqrt(2)) * complex(randn(1,channel_length),randn(1,channel_length)); ch2_time = gain(2)*(1/sqrt(2)) * complex(randn(1,channel_length),randn(1,channel_length)); data_due_channel = zeros(1,(length(cyclic_input)/channel_length)*(2*Cyclic_symbols-2)); indx = 1; SNR_range_linear = zeros(1,length(SNR_range_db)); % Received_data = zeros(length(SNR_range_db),length(cyclic_input)); for i= 1:length(SNR_range_db) SNR_range_linear(i) = 10 ^ (SNR_range_db(i)/10); for j = 1:channel_length:length(cyclic_input) if(mod(indx,2)==1) % Assign to channel 1 data_due_channel(j:j+channel_length-1) = cyclic_input(j:j+channel_length-1) .* ch1_time; Eavg = mean(abs(cyclic_input(j:j+channel_length-1)).^2); noise = sqrt(Eavg/(2*SNR_range_linear(i))) .* complex(randn(1,channel_length) , randn(1,channel_length)); Received_data(i,j:j+channel_length-1) = (data_due_channel(j:j+channel_length-1) + noise) ./ ch1_time; else % Assign to channel 2 data_due_channel(j:j+channel_length-1) = cyclic_input(j:j+channel_length-1) .* ch2_time; Eavg = mean(abs(cyclic_input(j:j+channel_length-1)).^2); noise = sqrt(Eavg/(2*SNR_range_linear(i))) .* complex(randn(1,channel_length) , randn(1,channel_length)); Received_data(i,j:j+channel_length-1) = (data_due_channel(j:j+channel_length-1) + noise) ./ ch2_time; end indx = indx +1; end end end %% -----------Decyclic extension Function------------------%% function decyclic_out = Decyclic(input_data,SNR_range_db) %decyclic_out = zeros(length(SNR_range_db),(Number_of_Bits/symbolBits)); % decyclic_out = zeros(length(SNR_range_db),size(input_data,2)); for j=1:length(SNR_range_db) for i=160:160:size(input_data,2) decyclic_out(j,((i/160)*128)-127:(i/160)*128)=input_data(j,i-127:i); end end end %% ----------------FFT Function (At Rx)----------------%% function out_data_fft = FFT(input_data,SNR_range_db) out_data_fft = zeros(length(SNR_range_db), size(input_data,2)); % Initialize output array for j = 1 : length(SNR_range_db) for i = 1:128:size(input_data,2) out_data_fft(j, i:i+127) = fft(input_data(j, i:i+127), 128); end end end %% ------------ DeMapper Function--------------------%% function demapped_data_binary = DeMapper (received_data ,SNR_range_db,symbolBits,Eb) demapped_data_binary = zeros(length(SNR_range_db),size(received_data,2)*symbolBits); demapped_data_decimal = zeros(length(SNR_range_db),size(received_data,2)); for i = 1 : length(SNR_range_db) switch symbolBits case 2 % QPSK case QPSK_table = sqrt(Eb) * [complex(-1,-1), complex(-1,1), complex(1,-1), complex(1,1)]; for j = 1 : size(received_data,2) [~ , Min_index] = min(abs(received_data(i,j) - QPSK_table)); demapped_data_decimal(i,j) = Min_index - 1; end demapped_data_binary(i,:) = reshape(de2bi(demapped_data_decimal(i,:),2,'left-msb')',1,[]); case 4 % 16-QAM case MQAM_table = sqrt(Eb)*[complex(-3,-3), complex(-3,-1), complex(-3,3), complex(-3,1), complex(-1,-3), complex(-1,-1), complex-1,3, complex-1,1, complex3,-3, complex3,-1, complex3,3, complex3,1,complex1,-3, complex(1,-1), complex(1,3), complex(1,1)]; for j = 1 : size(received_data,2) [~ , Min_index] = min(abs(received_data(i,j) - MQAM_table)); demapped_data_decimal(i,j) = Min_index - 1; end demapped_data_binary(i,:) = reshape(de2bi(demapped_data_decimal(i,:),4,'left-msb')',1,[]); end end end %% ---------------Deinterleaver Function-----------------%% function deintrlv_out = Deinterleaver(demapped_data,SNR_range_db,symbolBits) deintrlv_out = zeros(length(SNR_range_db), size(demapped_data,2)); % Initialize the output for j = 1 : length(SNR_range_db) for i = 1:(128*symbolBits):size(demapped_data,2) deintrlv_out(j, i:i+(128*symbolBits)-1) = matdeintrlv(demapped_data(j, i:i+(128*symbolBits)-1), symbolBits*8, 16); end end end %% --------------Pad Removal function (In case of Repition code)-----------%% function Deintrlv_final = pad_removal(Deintrlv_output,symbolBits) for i = 1:size(Deintrlv_output,1) k=1; if(symbolBits == 2) % QPSK ---> every 256 bits remove 1 bit from the end for j = 1:256:size(Deintrlv_output,2) frame = Deintrlv_output(i,j:j+255); Deintrlv_final(i,k:k+254) = frame(1:end-1); k=k+255; end else % 16-QAM ---> every 512 bits remove 2 bits from the end for j = 1:512:size(Deintrlv_output,2) frame = Deintrlv_output(i,j:j+511); Deintrlv_final(i,k:k+509) = frame(1:end-2); k=k+510; end end end end %%----------- Decoding Function ---------------- %% function [decoded_bitstream] = majority_decoder (encoded_bitstream , symbolBits ) hash_table = zeros(1,2^symbolBits); iterations = 0; j=1; decoded_bitstream = zeros(1,length(encoded_bitstream)/3); %% Assume 3-repeatition code for i = 1 : symbolBits : length(encoded_bitstream) iterations = iterations + 1 ; % Convert the symbol into the correspoding decimal into a hashtable % to see the no. of occurrences of such symbol for decision index = bin2dec(num2str((encoded_bitstream(i:i+symbolBits-1))))+1; hash_table(index) = hash_table(index) + 1; if(iterations==3) %decide based on the most no. of occurrences % then get the corrsponding binary symbol index_of_most_repeated_symbol=find(hash_table==max(hash_table),1); binaryStr = dec2bin(index_of_most_repeated_symbol-1, symbolBits); decoded_bitstream(j:j+symbolBits-1) = double(binaryStr)-'0'; j = j + symbolBits; hash_table = zeros(1,2^symbolBits); % Reset the hash table for the next set of symbols iterations = 0; end end end %% --------------BER Calculation function----------------%% function BER_value = compute_BER(received_data,Random_data,SNR_range_db) BER_value = zeros(1,length(SNR_range_db)); for i = 1 : length(SNR_range_db) BER_value(1,i) = length(find(xor(received_data(i,:),Random_data) == 1))/length(Random_data); end end

- After simulating the above code, we produce 4 curves as we have four possibilities (QPSK with flat channel, QPSK with frequency selective channel, 16-QAM with flat channel, 16-QAM with frequency selective channel)

Figure (7): BER of flat fading channel for QPSK modulation

Figure (8): BER of frequency selective fading channel for QPSK modulation

Figure (9): BER of flat fading channel for 16-QAM modulation

Figure (10): BER of frequency selective fading channel for 16-QAM modulation

- From the above graphs we notice that:

- Channel vs. Modulation Scheme:

Flat Fading Channel: For both QPSK and 16-QAM, the flat fading channel performs better overall compared to the frequency-selective channel. This is expected as flat fading introduces uniform distortion across all frequencies, which is easier to equalize.

Frequency-Selective Channel: This channel shows a more significant degradation in BER, especially for higher-order modulation (16-QAM). Frequency selectivity causes varying interference across different frequency components, making equalization harder and increasing BER.

- Effect of Coding with the Modulation Scheme on BER:

QPSK: Repetition coding demonstrates noticeable improvement in BER for both flat and frequency-selective channels. This shows the resilience of lower-order modulation when combined with simple error correction techniques like repetition coding.

16-QAM: Repetition coding still improves the BER, but the improvement is less pronounced compared to QPSK. This reflects the inherent vulnerability of higher-order modulation to noise and fading, which limits the coding gain.

- Insights on Modulation and Coding:

Higher-Order Modulation: As seen in 16-QAM, while it provides higher data rates, it is more sensitive to noise and fading. This trade-off makes it less effective in harsh channel conditions, even with coding.

Error-Correcting Codes: Simple coding techniques like repetition coding offer significant BER improvements, particularly for robust modulation schemes like QPSK. However, advanced coding methods may be needed for higher-order schemes to maximize performance gains.

- General Observations:

QPSK offers better BER under challenging conditions, 16-QAM provides higher data rates but demands more sophisticated error-correction strategies.